# O’tega Iteku & 20482233

# Project Planning Report for Software development of Drawing Robot

# Outline of the Problem to be Solved

This project intends to design a robotic system which takes the desire output of a user, written in English text, and have a robot write down what the user has typed into written, legible, English. To elablorate further, the user’s input will be stored in a sperate file called “TextData.txt” with a specified font for each text, stored in a separate file called  “SingleStrokeFont.txt”, this is then transferred as commands to a virtual port called “RS3232”, which leads to the Arduino MEGA unit, turning the commands into readable code for the robotic system to execute it’s intended function.

Furthermore, there are specific parameters that the code must fulfil;
1. The text created by the robotic system must be contained inside a 100mm box

2. Said text from the user’s input must be between 4mm to 10mm tall

Problems arise however that need to be solved for this project to be successful. Firstly, in the SingleStrokeFont.txt file, a group of functions must be developed to offset the individual letters in the x axis direction and y axis direction using the LF (ASCII 10) and CR  (ASCII 13), is to perfectly offset lines, but also a spacing of 5mm between each line. 

In addition, after developing the font through the SinlgeStrokeFont.txt file, it must be converted into G code for the writing arm to understand the commands, this is done through a virtual serial port (RS3232), where the writing arm almost forms a “queue” of G commands, taking each one by one, this being called a buffer.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| XOffset | float | Global position of the X axis in order to offset a single character when drawing with the robot arm |
| YOffset | float | Global position of the Y axis in order to offset a single character when drawing with the robot arm. |
| PenData | struct | Denotes the positions of the pen x and y axis relative to the (0,0) position, and also whether the pen is up or down |
| FontData | struct | Stores the Character numbers in ASCII Code and the number of movements need to write the character with the robotic arm |
| TextHeight | float | Contains the height of the text, determined by the user’s input, this would be validated by the program to ensure it is between 4mm to 10mm |
| FontMap | array | An array containing all the character data from SingleStrokeFont.txt file, with only one word from the statement |

Extend table as required

# Function Declarations

Float textScale(float textHeight * float 1/18);

Parameters:

textHeight – user’s input for the height of the text

1/18 – 18 is the number of units in the font file

Returning value – A to scale size of the user’s text height

char TransferGCode(const char*command);

Parameters:

Const char – data type for the commands given

Command – Placeholder term to signify the G code that will be type here

Returning value – Void in this case, but otherwise with the appropriate variables, a G-code would be sent the virtual serial port which would be queued, waiting for the “ok” signal from the arm

float StartRobotDrawing (G commands);

Parameters:

(G commands) – Where the initial robot G-code commands would be sent to turn on the spindle, set a writing speed and move the pen up

Returning value – Void in this case, but otherwise pen would move up, ask for a writing speed, and spindle would spool up

float EndRobotDrawing (G commands);

Parameters:

(G commands) – G code would be sent to the writing arm, resetting its position to zero

Returning value – In this case void, but usually would be a reset of the pen to position (0,0)

int ReadMesWord(Char FontMap[]);

Parameters:

ReadMesWord – Function name

CharData – Significant to make code read the SingleFontStroke quickly

FontMap[] – Reads FontMap

Returning value – Error

int OpenMesWord(const char * filename, char FontMap[], float textScale);

Parameters:

OpenMesWord – Function name

Const char – Sets data type for entire function

Filename – Asks for the name of the file, in the case of the program it would most likely be SingleStrokeFont.txt

Data char FontMap[] – Calls FontMap into the function, and inputs the characters from the file into the array

float textScale – Scales the text, ensuring that the words would be 5mm apart

Returning value – int

int DrawCharacter(char letter, Char Data FontMap[], float textScale, float *XOffset, float *YOffset);

Parameters:

DrawCharacter – Function name

Char letter – data type for the letter that would be converted into G code

Char Data FontMap[] – Calls the FontMap array

Float textScale - Scales the text to the user’s text height input

Float XOffset– Offsets character from the X axis

Float YOffset - Offsets character from the Y axis

Returning value – int

Char ReadNextWord(ReadMesWord++);

Parameters:

ReadMesWord – Function which would read the Fontdata for the first word in the “SingleFontStroke.txt” file

++ - Signifies moving to the next word within SingleStrokeFont.txt file by reading the next font data line

Returning value – char

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| textScale | textScale(textHeight/18); | False value = 11.0 True value = 8.0 | Error because textHeight is not between 4mm to 10mm Program will continue as textHeight is within 4 to 10 |
| DrawCharacter | Const char DrawCharacter(char letter, char FontMap[], float textScale, float *XOffset, float *YOffset); | False value = & True value = T | Error because char is not a letter in the English alphabet Outputs a set of G Code commands which would be taken to the “TransferGCode” function |
| OpenMesWord | OpenMesWord(const char * filename, char FontMap[]); | False value = “SingleBlockStroke.txt” True value = “SingleFontStroke.txt” | Error because that is not an existing file in the program Opens the file and transfer a word from file into the FontMap Array |
| ReadMesWord | int ReadMesWord(CharData FontMap[]); | False value = “FontSize[]” True value = “FontMap[]” | Error because this is not recognized as an array in the program Function would “read” the array FontMap, which is a single word from the SingleStrokeFont.txt file |
| StartRobotDrawing | StartRobotDrawing (G commands); | False value – T2203, 73, 91 True value – M3, F1000,S0, | Error because these are not recognised as true G code commands Turns on the spindle, sets the pen speed to 1000 mm min^-1 and pulls the pen upwards |
| EndRobotDrawing | EndRobotDrawing (G commands); | False value – T1 0 0, T1000 True value – G1 0 0, S0 | Error because these are not recognised as real G code commands Returns the pen to the (0,0) position in the coordinate space and pulls the pen upwards |
| TransferGCode | char TransferGCode(const char*G command); | False value – T0, T1 P0 F18 True value – S0 G0 X0 Y9 | Error because these are not recognised as G code commands Would pull the pen up and move it to the (0,9) coordinate space , using the font data (0 9 0) |
| ReadNextWord | Int ReadNextWord (ReadMesWord++); | True Value – ReadFontData++ False Value – ReadMesWord++ | Error as this is not recognised as a real function Would move to the next word within the “SingleStrokeFont.txt” file that is within the FontData[] array |

# Flowchart(s)

Will be included as separate PDFs